Rapid Embedded Systems

Design and Programming Course

Digital I/O

Issue 1.0

IC2002

CS200558-Duong Van Hoa

CE200736-Nguyen Quoc Kiet

Main Code

#include "mbed.h"

// Individual Digital Input pins for buttons

DigitalIn b1(D2);

DigitalIn b2(D3);

DigitalIn b3(D4);

DigitalIn b4(D5);

// Individual Digital Output pins for RGB LED

DigitalOut rled(D6);

DigitalOut gled(D7);

DigitalOut bled(D8);

// Bus configurations BusIn buttonsbus(D2, D3, D4, D5);

BusOut ledsbus(D6, D7, D8);

void ControlLED_DigitalIO(){

// RED LED logic: Active when (b4 is 0 AND (b3 is 0 OR b2 is 1)) OR

when all buttons are pressed

rled = (!b4 && (!b3 ¦¦ b2)) ¦¦ (b4 && b3 && b2 && b1);

// GREEN LED logic: Active when b4 and b3 are different (XOR), OR when

all buttons are pressed

gled = (b4 ^ b3) ¦¦ (b4 && b3 && b2 && b1);

// BLUE LED logic: Directly follows the state of button b4

bled = b4; }

void ControlLED_BusIO(){

switch (buttonsbus){

// Cases 0 to 3: Only RED LED is ON

case 0 ... 3: ledsbus = 0b001; // bled=0, gled=0, rled=1

break;

// Cases 4 to 5: Only GREEN LED is ON

case 4 ... 5:

ledsbus = 0b010; // bled=0, gled=1, rled=0

break;

// Cases 6 to 7: Both RED and GREEN LEDs are ON

case 6 ... 7:

ledsbus = 0b011; // bled=0, gled=1, rled=1

break;

// Cases 8 to 11: Both GREEN and BLUE LEDs are ON

case 8 ... 11: ledsbus = 0b110; // bled=1, gled=1, rled=0

break;

// Cases 12 to 14: Only BLUE LED is ON

case 12 ... 14:

ledsbus = 0b100; // bled=1, gled=0, rled=0

break;

// Case 15: All buttons pressed -> All LEDs are ON (White)

case 15:

ledsbus = 0b111; // bled=1, gled=1, rled=1

break;

default: ledsbus = 0b000; // Turn off all LEDs as a safety default

break;

}

}

int main()

{ // Infinite loop to continuously sample inputs and update outputs

while(1) {

// Toggle comments below to switch between execution methods

// ControlLED_DigitalIO();

ControlLED_BusIO();

}

}