EMBEDDED SYSTEMS PROGRAMMING WITH MBED OS

SPI Communication Between Two Boards

Master-to-Slave LED Control Using Two NUCLEO-F401RE Boards

| Item |  | Information |  |
| --- | --- | --- | --- |
| Course | Embedded Systems / Microcontroller Programming |
| Platform | 2 × STM32 NUCLEO-F401RE |
| Framework | Mbed OS |
| Lab Package | Lab 5.3 |
| Topic | SPI Master/Slave Communication, Inter-board Data Transfer, LED Control |
| Submission | Completed source code for both boards, answers, and evidence of circuit testing |

# Student Information

| Field |  | Student Input |  |
| --- | --- | --- | --- |
| Student Name | Nguyen Nhu Van Anh - Le Hieu Dat - Duong Van Hoa - Nguyen Quoc Kiet |
| Student ID | CE200171 - CE200509 - CS200558 - CE200736 |
| Class | IC2002 |
| Date | 02/07/2026 |

# General Submission Instructions

- Build the SPI circuit exactly as described in the connection table.

- Program one board as SPI Master and the other board as SPI Slave.

- Complete all missing code sections marked with blanks or TODO comments.

- Compile and run the programs on both STM32 NUCLEO-F401RE boards.

- Capture one photo of the circuit and one screenshot or short video showing the LED control result.

- Answer all lab questions using short, clear explanations.

## Important Safety Note

Always connect a common GND between the two boards. Connect MOSI to MOSI, MISO to MISO, SCK to SCK, and CS from the Master to the Slave chip-select input. Do not connect LEDs without 220Ω–330Ω current-limiting resistors.

SPI Communication Between Two Boards

## 1. Objective

In this lab, students implement an SPI communication link between two STM32

NUCLEO-F401RE boards. One board acts as the SPI Master (controller) and sends one-byte command data to the second board, which acts as the SPI Slave (target). The Slave board uses the received byte to control three LEDs. The goal is to understand how synchronous serial communication works and how the Master controls the transfer timing using the SPI clock.

## 2. Learning Outcomes

·       Create an SPI interface on the STM32 NUCLEO-F401RE.

·       Understand the function of MOSI, MISO, SCK, and CS lines.

·       Configure one board as SPI Master and another as SPI Slave.

·       Send one-byte control data from the Master board.

·       Receive control data on the Slave board.

·       Use received SPI data to control LEDs.

·       Print debug information to the serial terminal using printf().

## 3. Required Hardware

| Component | Quantity |  | Purpose |
| --- | --- | --- | --- |

STM32 NUCLEO-F401RE 	2 	One board as Master, one

board as Slave

LED 	3 	Visual output on the Slave

board

220Ω–330Ω resistor 	3 	Current limiting for LEDs

Breadboard 	2 or 1 large 	Circuit assembly

Jumper wires 	Several 	Connections between

boards and LEDs

USB cable 	2 	Board power and serial

terminal access

## 4. Hardware Setup / Circuit Connections

| Connection | Master Board | Slave Board |  | Description |
| --- | --- | --- | --- | --- |
| SPI MOSI | D11 | D11 |  | Master Out Slave In data line |
| SPI MISO | D12 | D12 |  | Slave Out Master In data line |
| SPI SCK | D13 | D13 |  | SPI clock line |
| Chip Select (CS) | D10 | D10 input / chip select |  | Master selects the Slave before transfer |
| Common Ground | GND | GND |  | Required common reference |
| Red LED | - | D5 via 220Ω–330Ω |  | Turns ON when command A is received |
| Green LED | - | D6 via 220Ω–330Ω |  | Turns ON when command B is received |
| Blue LED | - | D7 via 220Ω–330Ω |  | Turns ON when command C is received |
| Simple wiring view Master D11 (MOSI) -------- Slave D11 (MOSI) Master D12 (MISO) -------- Slave D12 (MISO) Master D13 (SCK) -------- Slave D13 (SCK) Master D10 (CS) -------- Slave D10 (CS) Master GND -------- Slave GND Slave D5 ---- 330Ω ---- Red LED ---- GND Slave D6 ---- 330Ω ---- Green LED ---- GND Slave D7 ---- 330Ω ---- Blue LED ---- GND 5. Functional Requirements |  |  |  |  |
| Board / I/O Pin / Interface |  |  | Required Behavior |  |

Master Board 	D11/D12/D13/D10 	Send one command byte

over SPI.

Slave Board 	D11/D12/D13/D10 	Receive the command byte

from the Master.

Slave Red LED 	D5 	Turn ON when command A

is received.

Slave Green LED 	D6 	Turn ON when command B

is received.

Slave Blue LED 	D7 	Turn ON when command C

is received.

Serial terminal 	USBTX/USBRX 	Display sent/received

debugging messages.

## 6. Key Theory

| Concept | In this Lab | Meaning |
| --- | --- | --- |

SPI bus 	MOSI + MISO + SCK + CS 	Four-wire synchronous

serial communication

between boards.

Master 	SPI master(D11, D12, D13); 	Initiates transfer and

generates the SPI clock.

Slave 	Connected to 	Receives data only when

MOSI/MISO/SCK/CS 	selected by CS.

Chip Select 	DigitalOut cs(D10); 	Selects the Slave before

data transfer.

DigitalOut 	redLed(D5); 	Used to control LEDs based

on received data.

Received command 	A / B / C 	Selects which LED should

turn ON.

## 7. Starter Code - Master Board (Fill in the Missing Parts)

#include "mbed.h"

#include <cstdio>

#define MOSI_PIN D11

#define MISO_PIN D12

#define SCK_PIN  D13

#define CS_PIN   D10

// Khởi tạo SPI Master và chân Chip Select (CS)

SPI master(MOSI_PIN, MISO_PIN, SCK_PIN);

DigitalOut cs(CS_PIN);

char tx;

int main()

{

master.format(8, 0);     master.frequency(100000);

cs = 1;

printf("--- SPI Master Ready ---\r\n");     printf("Type A, B, or C in the serial terminal to control Slave LEDs\r\n");     while (true)

{

// Đọc ký tự từ Serial Terminal         char cmd = getchar();

// Bỏ qua các ký tự xuống dòng         if (cmd == '\n' || cmd == '\r')

{             continue;

}

// Lưu ký tự vào biến truyền         tx = cmd;                cs = 0;

ThisThread::sleep_for(1ms); // Chờ một chút ngắn để Slave kịp đáp ứng

// Truyền 1 byte dữ liệu qua SPI         master.write(tx);

// Hủy kích hoạt Slave (Kéo chân CS lên mức Cao)         ThisThread::sleep_for(1ms);

cs = 1;

printf("Master sent: %c\r\n", tx);

}

}

## 8. Starter Code - Slave Board (Fill in the Missing Parts)

#include "mbed.h"

#include <cstdio>

#define RED_LED   D5

#define GREEN_LED D6

#define BLUE_LED  D7

// Khởi tạo các chân Output cho LED

DigitalOut redLed(RED_LED);

DigitalOut greenLed(GREEN_LED);

DigitalOut blueLed(BLUE_LED);

// Khởi tạo SPI Slave (Bỏ qua chân CS phần cứng bằng NC để tránh lỗi kẹt xung đột) SPISlave slave(D11, D12, D13, NC);

char rx; int main()

{         slave.format(8, 0);     redLed = 0;     greenLed = 0;     blueLed = 0;

printf("--- SPI Slave Ready ---\r\n");

while (true)

{

// Kiểm tra xem có tín hiệu SPI truyền đến hay không         if (slave.receive())

{

// Đọc 1 byte dữ liệu nhận được từ Master             rx = slave.read();

printf("Slave received: %c\r\n", rx);

// Điều khiển LED dựa trên ký tự nhận được

if (rx == 'A' || rx == 'a')

{                 redLed = 1;

greenLed = 0;                 blueLed = 0;

}             else if (rx == 'B' || rx == 'b')

{                 redLed = 0;                 greenLed = 1;                 blueLed = 0;

}             else if (rx == 'C' || rx == 'c')

{

redLed = 0;                 greenLed = 0;                 blueLed = 1;

}             else

{

// Nếu nhận ký tự khác, tắt hết LED                 redLed = 0;                 greenLed = 0;                 blueLed = 0;

}

}

}

}

## 9. Example Format

SPI master(D11, D12, D13);  DigitalOut cs(D10);  master.format(8, 0);  master.frequency(1000000);  cs = 0;  master.write(tx);  cs = 1;

SPISlave slave(D11, D12, D13, D10);  if (slave.receive()) rx = slave.read();

These examples show how to create SPI master/slave interfaces, configure the bus, select the Slave, and transfer one byte.

## 10. Expected Observation

☐ When the Master sends A, the Slave red LED turns ON.

☐ When the Master sends B, the Slave green LED turns ON.

☐ When the Master sends C, the Slave blue LED turns ON.

☐ The Master terminal prints the sent character.

☐ The Slave terminal prints the received character.

☐ The two boards communicate through MOSI, MISO, SCK, CS, and GND.

## 11. Student Questions

- What is the role of MOSI in SPI communication?

Answer: MOSI (Master Out, Slave In) is the data line used to send data from the Master to the Slave.

- What is the role of MISO in SPI communication?

Answer: MISO (Master In, Slave Out) is the data line used to send data from the Slave to the Master.

- What is the role of SCK in SPI communication?

Answer: SCK (Serial Clock) is the clock signal generated by the Master to synchronize data transfer between the Master and the Slave.

- Why is the CS line required in SPI communication?

Answer: The CS (Chip Select) line is used to select which Slave device the Master wants to communicate with. Only the selected Slave responds to the communication.

- What is the difference between an SPI Master and an SPI Slave?

Answer: The Master controls the communication by generating the clock signal and selecting the Slave. The Slave responds only when its CS line is activated by the Master.

- What happens when the Slave receives the character A?

Answer: When the Slave receives the character 'A', it performs the programmed action, such as turning on an LED or executing another predefined task.

- What is the purpose of slave.receive() in the SPI Slave code?

Answer: slave.receives() checks whether the Slave has received data from the Master. It lets the program know when new data is available to read.

- Why must both boards use the same SPI format and mode?

Answer: Both boards must use the same SPI format and mode to ensure that data is sampled and transmitted at the correct clock timing. If the settings are different, the communication may fail or produce incorrect data.

## 12. Suggested Test Procedure

- Power both NUCLEO boards using separate USB cables.

- Verify that MOSI, MISO, SCK, CS, and GND are correctly connected between the two boards.

- Open a serial terminal for the Master board and a serial terminal for the Slave board.

- Type A on the Master terminal and observe the Slave red LED.

- Type B on the Master terminal and observe the Slave green LED.

- Type C on the Master terminal and observe the Slave blue LED.

- Check the printed debugging messages on both terminals.

## 13. Challenge Tasks

·       Add command O so that the Master can turn all Slave LEDs OFF.

·       Modify the Slave so that it sends one status byte back to the Master using MISO.

·       Expand the protocol so digits 1, 2, 3 select blink patterns instead of simple ON/OFF.

·       Use a second SPI Slave and add another chip-select line on the Master.

# Appendix A - Mbed Commands to Remember

| Command / Syntax |  | Used For | Example | Important Note |  |
| --- | --- | --- | --- | --- | --- |
| SPI master(mosi, miso, sclk); | Create SPI Master interface | SPI master(D11, D12, D13); | Used on the board that starts communication. |
| master.format(bits, mode); | Set SPI data format | master.format(8, 0); | Both Master and Slave must use the same format. |
| master.frequency(hz) ; | Set SPI clock speed | master.frequency(100000 0); | 1 MHz is common in simple labs. |
| master.write(value); | Send one byte over SPI | master.write(tx); | Returns the byte received from the Slave at the same time. |
| SPISlave slave(mosi, miso, sclk, ssel); | Create SPI Slave interface | SPISlave slave(D11, D12, D13, D10); | Used on the receiving board. |
| slave.receive(); | Check for SPI event | if (slave.receive()) | Detects whether the Slave has been selected and data is available. |
| slave.read(); | Read byte from SPI bus | rx = slave.read(); | Reads the received byte. |
| DigitalOut led(pin); | Create LED output object | DigitalOut redLed(D5); | Used to control LEDs. |

# Appendix B - Key Concept Summary

| Concept |  | Code Example | Meaning |
| --- | --- | --- | --- |

Master initialization 	SPI master(D11, D12, D13); 	Creates the controller side of the SPI bus.

| Slave initialization | SPISlave slave(D11, D12, D13, D10); | Creates the target side of the SPI bus. |
| --- | --- | --- |
| Chip Select | cs = 0; ... cs = 1; | The Master selects the Slave before transfer and releases it afterward. |
| Send one byte | master.write(tx); | Transfers one command byte from Master to Slave. |
| Receive one byte | rx = slave.read(); | Reads the command byte on the Slave board. |
| LED action | if (rx == 'A') redLed = 1; | Maps received data to LED behavior. |

# Appendix C - Suggested Grading Rubric

| Category |  | Score | Criteria |  |
| --- | --- | --- | --- | --- |
| Circuit wiring | 20 | Correct MOSI, MISO, SCK, CS, GND, and LED wiring; neat breadboard layout. |
| Code completion | 35 | All blanks completed correctly for both Master and Slave; programs compile and run. |
| Lab behavior | 25 | Master sends data correctly; Slave receives data and controls LEDs correctly. |
| Questions | 10 | Answers are clear and technically correct. |
| Presentation | 10 | Submission is clean, complete, and includes evidence of testing. |