EMBEDDED SYSTEMS PROGRAMMING WITH MBED OS

I2C Communication Between Two Boards

Master-to-Slave LED Control Using Two NUCLEO-F401RE Boards

| Item |  | Information |  |
| --- | --- | --- | --- |
| Course | Embedded Systems / Microcontroller Programming |
| Platform | 2 × STM32 NUCLEO-F401RE |
| Framework | Mbed OS |
| Lab Package | Lab 5.2 |
| Topic | I2C Master/Slave Communication, LED Control, Inter-board Data Transfer |
| Submission | Completed source code for both boards, answers, and evidence of circuit testing |

# Student Information

| Field |  | Student Input |  |
| --- | --- | --- | --- |
| Student Name | Nguyen Nhu Van Anh - Le Hieu Dat - Duong Van Hoa - Nguyen Quoc Kiet |
| Student ID | CE200171 - CE200509 - CS200558 - CE200736 |
| Class | IC2002 |
| Date | 02/07/2026 |

# General Submission Instructions

- Build the I2C circuit exactly as described in the connection table.

- Program one board as I2C Master and the other board as I2C Slave.

- Complete all missing code sections marked with blanks or TODO comments.

- Compile and run the programs on both STM32 NUCLEO-F401RE boards.

- Capture one photo of the circuit and one screenshot or short video showing the LED control result.

- Answer all lab questions using short, clear explanations.

Important Safety Note: Always connect a common GND between the two boards. Connect SDA to SDA and SCL to SCL. Do not swap these lines. If pull-up resistors are not provided on the boards or by the circuit, add suitable pull-up resistors to the I2C lines.

I2C Communication Between Two Boards

## 1. Objective

:In this lab, students implement an I2C communication link between two STM32

NUCLEO-F401RE boards. One board acts as the I2C Master (controller) and sends a command byte to the second board, which acts as the I2C Slave (target). The Slave board uses the received data to control LEDs. The goal is to understand how two embedded boards exchange data over the I2C bus.

## 2. Learning Outcomes

·       Create an I2C interface on the STM32 NUCLEO-F401RE.

·       Understand the function of SDA and SCL lines.

·       Configure one board as I2C Master and another as I2C Slave.

·       Send one-byte control data from the Master board.

·       Receive control data on the Slave board.

·       Use received I2C data to control LEDs.

·       Print debug information to the serial terminal using printf() or Serial.print().

## 3. Required Hardware

| Component | Quantity |  | Purpose |
| --- | --- | --- | --- |

STM32 NUCLEO-F401RE 	2 	One board as Master, one

board as Slave

LED 	3 	Visual output on the Slave

board

220Ω–330Ω resistor 	3 	Current limiting for LEDs

Breadboard 	2 or 1 large 	Circuit assembly

Jumper wires 	Several 	Connections between

boards and LEDs

USB cable 	2 	Board power and serial

terminal access

## 4. Hardware Setup / Circuit Connections

| Connection | Master Board | Slave Board | Description |
| --- | --- | --- | --- |
| I2C SDA | D14 | D14 | Data line of the I2C bus |
| I2C SCL | D15 | D15 | Clock line of the I2C bus |
| Common Ground | GND | GND | Required common reference |
| Red LED | - | D5 via 220Ω–330Ω | Turns ON when command A is received |
| Green LED | - | D6 via 220Ω–330Ω | Turns ON when command B is received |
| Blue LED | - | D7 via 220Ω–330Ω | Turns ON when command C is received |
| Simple wiring view Master D14 (SDA) -------- Slave D14 (SDA) Master D15 (SCL) -------- Slave D15 (SCL) Master GND -------- Slave GND Slave D5 ---- 330Ω ---- Red LED ---- GND Slave D6 ---- 330Ω ---- Green LED ---- GND Slave D7 ---- 330Ω ---- Blue LED ---- GND 5. Functional Requirements |  |  |  |
| Board / I/O Pin / Address |  | Required Behavior |  |

Master Board 	D14/D15 	Send one command byte

over I2C.

Slave Board 	Address 0x20 	Receive the command byte

from the Master.

Slave Red LED 	D5 	Turn ON when command A

is received.

Slave Green LED 	D6 	Turn ON when command B

is received.

Slave Blue LED 	D7 	Turn ON when command C

is received.

Serial terminal 	USBTX/USBRX or Arduino 	Display sent/received

Serial 	debugging messages.

## 6. Key Theory

| Concept | In this Lab | Meaning |
| --- | --- | --- |

I2C bus 	SDA + SCL 	Two-wire serial

communication between

boards.

Master 	I2C master(D14, D15); 	Initiates communication and

sends data.

Slave 	Address 0x20 	Waits for the Master and

responds to received data.

Slave address 	0x20 	Unique address used by the

Master to reach the Slave.

DigitalOut 	redLed(D5); 	Used to control LEDs based

on received data.

Received command 	A / B / C 	Selects which LED should

turn ON.

## 7. Starter Code - Master Board (Fill in the Missing Parts)

#include "mbed.h"

#include <cstdio>

#define SDA_PIN D14

#define SCL_PIN D15

I2C master(SDA_PIN, SCL_PIN); const int SLAVE_ADDR = 0x20 << 1;

char tx;  int main()

{

master.frequency(100000);

printf("I2C Master Ready...\r\n");         printf("Type A, B, or C in the serial terminal\r\n");

while (true)

{

char cmd = getchar();         if (cmd == '\n' || cmd == '\r')

{                         continue;

}

// Lưu ký tự hợp lệ vào biến truyền tx         tx = cmd;         int ack = master.write(SLAVE_ADDR, &tx, 1);         if (ack == 0) {             printf("Master sent: %c (ACK received)\r\n", tx);

} else {

printf("Master sent: %c (NACK/Error - No response from Slave!)\r\n", tx);

}

}

}

## 8. Starter Code - Slave Board (Fill in the Missing Parts)

#include "mbed.h"

#include <cstdio>

// Định nghĩa các chân điều khiển LED trên Slave Board

#define RED_LED   D5

#define GREEN_LED D6

#define BLUE_LED  D7

DigitalOut redLed(RED_LED);

DigitalOut greenLed(GREEN_LED);

DigitalOut blueLed(BLUE_LED);

I2CSlave slave(D14, D15);

char rx; int main()

{

redLed = 0;     greenLed = 0;         blueLed = 0;        slave.address(0x20 << 1);

printf("I2C Slave Ready...\r\n");

while (true)

{    int status = slave.receive();         if (status == I2CSlave::WriteAddressed)

{

slave.read(&rx, 1);    	printf("Slave received: %c\r\n", rx);

if (rx == 'A' || rx == 'a')

{                                 redLed = 1;                   greenLed = 0;                                 blueLed = 0;

}                         else if (rx == 'B' || rx == 'b')

{                                 redLed = 0;                 greenLed = 1;                 blueLed = 0;

}                         else if (rx == 'C' || rx == 'c')

{

redLed = 0;                 greenLed = 0;                                 blueLed = 1;

}                         else                         {   redLed = 0;                 greenLed = 0;                                 blueLed = 0;

}

}

}

}

## 9. Example Format

I2C master(D14, D15);  const int SLAVE_ADDR = 0x20 << 1;  master.write(SLAVE_ADDR, &tx, 1);  I2CSlave slave(D14, D15);  slave.address(0x20 << 1);  slave.read(&rx, 1);

These examples show how to create I2C master/slave interfaces, set the address, and transfer one byte.

## 10. Expected Observation

☐ When the Master sends A, the Slave red LED turns ON.

☐ When the Master sends B, the Slave green LED turns ON.

☐ When the Master sends C, the Slave blue LED turns ON.

☐ The Master terminal prints the sent character.

☐ The Slave terminal prints the received character.

☐ The two boards communicate through SDA and SCL only, with a common GND.

## 11. Student Questions

- What is the role of SDA in I2C communication?

Answer: SDA (Serial Data) is the data line used to transmit and receive data between the Master and the Slave devices.

- What is the role of SCL in I2C communication?

Answer: SCL (Serial Clock) is the clock line generated by the Master to synchronize data transmission on the SDA line.

- Why must the two boards share a common GND?

Answer: A common GND provides the same voltage reference for both boards, ensuring reliable communication and correct signal interpretation.

- What is the difference between an I2C Master and an I2C Slave?

Answer: The Master controls the communication by generating the clock and initiating data transfer. The Slave responds only when its address is called by the Master.

- Why does the Master use a slave address?

Answer: The Master uses the slave address to identify and communicate with a specific Slave device on the I2C bus.

- What happens when the Slave receives the character A?

Answer: When the Slave receives the character 'A', it executes the programmed action, such as turning on an LED or performing another predefined task.

- What is the purpose of slave.receive()?

Answer: slave.receive() checks whether the Slave has received data from the Master. It allows the program to detect incoming communication before reading the data.

- Why is the slave address often written as 0x20 << 1 in Mbed examples?

Answer: 0x20 << 1 shifts the 7-bit I2C address left by one bit to create the 8-bit address format used by the Mbed I2C library. The least significant bit is reserved for the read/write operation.

## 12. Suggested Test Procedure

- Power both NUCLEO boards using separate USB cables.

- Verify that SDA, SCL, and GND are correctly connected between the two boards.

- Open a serial terminal for the Master board and a serial terminal for the Slave board.

- Type A on the Master terminal and observe the Slave red LED.

- Type B on the Master terminal and observe the Slave green LED.

- Type C on the Master terminal and observe the Slave blue LED.

- Check the printed debugging messages on both terminals.

## 13. Challenge Tasks

·       Add command O so that the Master can turn all Slave LEDs OFF.

·       Modify the Slave so that it sends an ACK text back to the Master.

·       Expand the protocol so digits 1, 2, 3 select blink patterns instead of simple ON/OFF. ·       Use a second Slave board with a different I2C address.

## 14. Image

# Appendix A - Mbed Commands to Remember

| Command / Syntax | Used For | Example | Important Note |
| --- | --- | --- | --- |

I2C master(sda, scl); 	Create I2C 	I2C master(D14, D15); 	Used on the board

Master interface 	that starts

communication.

| master.frequency(hz) ; | Set I2C bus speed | master.frequency(100000 ); | 100 kHz is common for simple labs. |
| --- | --- | --- | --- |
| master.write(addr, buf, len); | Send bytes to Slave | master.write(SLAVE_AD DR, &tx, 1); | Writes one or more bytes. |
| I2CSlave slave(sda, scl); | Create I2C Slave interface | I2CSlave slave(D14, D15); | Used on the receiving board. |
| slave.address(addr); | Set Slave address | slave.address(0x20 << 1); | Address must match the Master target address. |
| slave.receive(); | Check for I2C event | int status = slave.receive(); | Detects whether the Slave has been addressed for write or read. |
| slave.read(&rx, 1); | Read data from Master | slave.read(&rx, 1); | Reads one byte into rx. |
| DigitalOut led(pin); | Create LED output object | DigitalOut redLed(D5); | Used to control LEDs. |

# Appendix B - Key Concept Summary

| Concept | Code Example | Meaning |
| --- | --- | --- |

Master initialization 	I2C master(D14, D15); 	Creates the controller side of the I2C bus.

| Slave initialization | I2CSlave slave(D14, D15); | Creates the target side of the I2C bus. |
| --- | --- | --- |
| Slave address | slave.address(0x20 << 1); | Identifies the Slave board on the I2C bus. |
| Send one byte | master.write(SLAVE_ADDR, &tx, 1); | Transfers one command byte from Master to Slave. |
| Receive one byte | slave.read(&rx, 1); | Reads the command byte on the Slave board. |
| LED action | if (rx == 'A') redLed = 1; | Maps received data to LED behavior. |

# Appendix C - Suggested Grading Rubric

| Category |  | Score | Criteria |  |
| --- | --- | --- | --- | --- |
| Circuit wiring | 20 | Correct SDA, SCL, GND, and LED wiring; neat breadboard layout. |
| Code completion | 35 | All blanks completed correctly for both Master and Slave; programs compile and run. |
| Lab behavior | 25 | Master sends data correctly; Slave receives data and controls LEDs correctly. |
| Questions | 10 | Answers are clear and technically correct. |
| Presentation | 10 | Submission is clean, complete, and includes evidence of testing. |